B.M.S College of Engineering

(Autonomous Institution affiliated to VTU, Belagavi)

Bengaluru – 19

Department of Computer Science and Engineering

Report on

“Verilog Programs using Structural Modeling, Behavioral Modeling and Data Flow Modeling”

LOGIC DESIGN - 19CS3PCLOD

(Autonomous Scheme 2020-21)

Submitted by

Name: NITHIN.C

USN: 1BM19CS106

B.M.S. College of Engineering

(Autonomous Institution affiliated to VTU, Belagavi)

Bengaluru - 19

Department of Computer Science and Engineering

Certificate

This is to certify that Mr. __ NITHIN.C_ has satisfactorily completed the course of Experiments in LOGIC DESIGN course prescribed by the Department during the year 2020-2021.

Name of the Candidate: NITHIN.C

USN No:1BM19CS106           Semester: 3

| Marks |  |  |
| --- | --- | --- |
| Max. Marks | Obtained |
| 20 |  |

| Marks in Words |  |
| --- | --- |

Signature of the staff in-charge  	                                   Head of the Department

Date:

Verilog Program List

19CS3PCLOD

| Serial No. | Title |
| --- | --- |
|  | CYCLE I Structural Modeling |
|  | Write HDL implementation for the following Logic - AND/OR/NOT Simulate the same using structural model and depict the timing diagram for valid inputs. |
|  | Write HDL implementation for the following Logic - NAND/NOR Simulate the same using structural model and depict the timing diagram for valid inputs. |
|  | Write HDL implementation for the following Logic Simulate the same using structural model and depict the timing diagram for valid inputs. |
|  | Write HDL implementation for a 4:1 Multiplexer. Simulate the same using structural model and depict the timing diagram for valid inputs. |
|  | Write HDL implementation for a 2-to-4 decoder. Simulate the same using structural model and depict the timing diagram for valid inputs. |
|  | Write HDL implementation for a 4-to-2 encoder. Simulate the same using structural model and depict the timing diagram for valid inputs. |
|  | CYCLE II Behavior Modeling |
|  | Write HDL implementation for a RS flip-flop using behavioral model. Simulate the same using Behavior model and depict the timing diagram for valid inputs. |
|  | Write HDL implementation for a JK flip-flop using behavioral model. Simulate the same using Behavior model and depict the timing diagram for valid inputs. |
|  | Write HDL implementation for a 4-bit right shift register using behavioral model. Simulate the same using Behavior model and depict the timing diagram for valid inputs. |
|  | Write HDL implementation for a 3-bit up-counter using behavioral model. Simulate the same using Behavior model and depict the timing diagram for valid inputs. |
|  | CYCLE III Dataflow Modeling |
|  | Write HDL implementation for AND/OR/NOT gates using data flow model. Simulate the same using Dataflow model and depict the timing diagram for valid inputs. |
|  | Write HDL implementation for a 3-bit full adder using data flow model. Simulate the same using Dataflow model and depict the timing diagram for valid inputs. |

STRUCTURAL MODELING

Experiment 1:

- Write HDL implementation for the following Logic

- AND/OR/NOT

Simulate the same using structural model and depict the timing diagram for valid inputs.

Main Module:

module s_andornot(a,b,y);

input a,b;

output [2:0]y;

and a(y[2],a,b);

or o(y[1],a,b);

not n(y[0],a);

endmodule

Test Module:

module tb_andornot;

reg a,b;

wire [2:0]y;

s_andornot ob(a,b,y);

initial

begin

$dumpfile("test.vcd");

$dumpvars(0,tb_andornot);

a=1'b0;b=1'b0;

#20;

a=1'b0;b=1'b1;

#20;

a=1'b1;b=1'b0;

#20;

a=1'b1;b=1'b1;

#20;

$finish;

end

endmodule

Experiment 2:

Write HDL implementation for the following Logic

- NAND/NOR

Simulate the same using structural model and depict the timing diagram for valid inputs.

Main Module:

module s_nandnor(a,b,y);

input a,b;

output [1:0]y;

nand a(y[1],a,b);

nor o(y[0],a,b);

endmodule

Test Module:

`include "s_nandnor.v"

module tb_nandnor;

reg a,b;

wire [1:0]y;

s_nandnor ob(a,b,y);

initial

begin

$dumpfile("test.vcd");

$dumpvars(0,tb_nandnor);

a=1'b0;b=1'b0;

#20

a=1'b0;b=1'b1;

#20

a=1'b1;b=1'b0;

#20

a=1'b1;b=1'b1;

#20

$finish;

End

endmodule

Experiment 3:

Write HDL implementation for the following Logic

Simulate the same using structural model and depict the timing diagram for valid inputs.

Main Module:

module andor(A,B,C,D,Y);

input A,B,C,D;

output Y;

wire al,a2;

and gl(al,A,B);

and g2(a2,C,D);

or g3(Y,al,a2);

endmodule

Test Module:

`include "andor.v"

module test_andor;

reg a,b,c,d;

wire y;

andor ao(a,b,c,d,y);

initial

begin

$dumpfile("andor.vcd");

$dumpvars(0,test_andor);

a=0; b=1; c=1; d=1; #10

a=0; b=0; c=1; d=0; #10

$finish;

end

endmodule

Experiment 4:

Write HDL implementation for a 4:1 Multiplexer. Simulate the same using structural model and depict the timing diagram for valid inputs.

Main Module:

module mux4to1(a, b, c, d, sel1, sel2, y);

input a, b, c, d, sel1, sel2;

output y;

wire and1, and2, and3, and4;

and g1(and1, a, ~sel1, ~sel2);

and g2(and2, b, ~sel1, sel2);

and g3(and3, c, sel1, ~sel2);

and g4(and4, d, sel1, sel2);

or g5(y, and1, and2, and3, and4);

endmodule

Test Module:

module test;

reg a, b, c, d, sel1, sel2;

wire y;

mux4to1 mg(a, b, c, d, sel1, sel2, y);

initial

begin

$dumpfile("mux4to1.vcd");

$dumpvars(0, test);

a = 0; b = 0; c = 0; d = 0; sel1 = 0; sel2 = 0;

#20

a = 1; b = 0; c = 0; d = 0; sel1 = 0; sel2 = 0;

#20

a = 0; b = 0; c = 0; d = 0; sel1 = 0; sel2 = 1;

#20

a = 0; b = 1; c = 0; d = 0; sel1 = 0; sel2 = 1;

#20

a = 0; b = 0; c = 0; d = 0; sel1 = 1; sel2 = 0;

#20

a = 0; b = 0; c = 1; d = 0; sel1 = 1; sel2 = 0;

#20

a = 0; b = 0; c = 0; d = 0; sel1 = 1; sel2 = 1;

#20

a = 0; b = 0; c = 0; d = 1; sel1 = 1; sel2 = 1;

#20

$finish;

end

endmodule

Experiment 5:

Write HDL implementation for a 2-to-4 decoder. Simulate the same using structural model and depict the timing diagram for valid inputs.

Main Module:

module decoder(Do,Din,En);

input En;

input[1:0]Din;

output [3:0]Do;

reg [3:0]Do;

always@(En or Din)

begin

if(En)

begin

case(Din)

2'b00:Do=4'b0001;

2'b01:Do=4'b0010;

2'b10:Do=4'b0100;

2'b11:Do=4'b1000;

default:Do=4'bzzzz;

endcase

end

end

endmodule

Test Module:

`include "decoder.v"

module test_dec;

reg [1:0]Din;

reg En;

wire [3:0]Do;

decoder  udue(.Do(Do),.Din(Din),.En(En));

initial begin

$dumpfile("decoder.vcd");

$dumpvars(0,test_dec);

En=1;

Din=2'b00;#100;

Din=2'b01;#100;

Din=2'b10;#100;

Din=2'b11;#100;

end

endmodule

Experiment 6:

Write HDL implementation for a 4-to-2 encoder. Simulate the same using structural model and depict the timing diagram for valid inputs.

Main Module:

module encoder(Do,Din,En);

input En;

input[3:0]Din;

output [1:0]Do;

reg [1:0]Do;

always@(En or Din)

begin

if(En)

begin

case(Din)

4'b0001:Do=2'b00;

4'b0010:Do=2'b01;

4'b0100:Do=2'b10;

4'b1000:Do=2'b11;

default:Do=2'bzz;

endcase

end

end

endmodule

Test Module:

`include "encoder.v"

module test_encoder;

reg [3:0]Din;

reg En;

wire [1:0]Do;

encoder  mux1(.Do(Do),.Din(Din),.En(En));

initial begin

$dumpfile("encoder.vcd");

$dumpvars(0,test_encoder);

En=1;

Din=4'b0001;#100;

Din=4'b0010;#100;

Din=4'b0100;#100;

Din=4'b1000;#100;

end

endmodule

BEHAVIOUR MODELING

Experiment 7:

Write HDL implementation for a SR flip-flop using behavioral model. Simulate the same using behavioral model and depict the timing diagram for valid inputs.

Main Module:

module SR_FF (sr, clk, q, qb);

input [1:0] sr;

input clk;

output reg q=1'b0;

output reg qb;

always @ (posedge clk)

begin

case (sr)

2'b00 : q = q ;

2'b01 : q = 1'b0 ;

2'b10 : q = 1'b1 ;

2'b11 : q = 1'bz ;

endcase

qb =~ q;

end

endmodule

Test Module:

module testsrflipf;

reg [1:0] A;

reg c;

wire x, xb;

SR_FF srff(A,c,x,xb);

initial c=1'b0;

always #5 c=~c;

initial

begin

A=2'b00; #10

A=2'b01;#10

A=2'b10;#10

A=2'b11;

#20 $finish;

end

endmodule

Experiment 8:

Write HDL implementation for a JK flip-flop using behavioral model. Simulate the same using behavioral model and depict the timing diagram for valid inputs.

Main Module:

module jk_flipflop1(j,k,clk,reset,q,q_bar);

input j,k,clk,reset;

output q,q_bar;

wire j,k,clk,reset;

reg q,q_bar;

always@(posedge clk)begin

if(reset)begin

q=1'b0;

q_bar=1'b1;

end else begin

case({j,k})

{1'b0,1'b0}:begin q=q;q_bar=q_bar;end

{1'b0,1'b1}:begin q=1'b0;q_bar=1'b1;end

{1'b1,1'b0}:begin q=1'b1;q_bar=1'b0;end

{1'b1,1'b1}:begin q=~q;q_bar=~q_bar;end

endcase

end

end

endmodule

Test Module:

`include "jk_flipflop1.v"

module JK_ff_tb;

reg clk;

reg reset;

reg j,k;

wire q;

wire qb;

jk_flipflop1 flipflop(.clk(clk),.reset(reset),.j(j),.k(k),.q(q),.q_bar(qb));

initial begin

$dumpfile("jk.vcd");

$dumpvars(0,JK_ff_tb);

$monitor(clk,j,k,q,qb,reset);

j=1'b0;

k=1'b0;

reset=1;

clk=1;

#10

reset=0;

j=1'b1;

k=1'b0;

#100

reset=0;

j=1'b0;

k=1'b1;

#100

reset=0;

j=1'b1;

k=1'b1;

#100

reset=0;

j=1'b0;

k=1'b0;

#100

reset=1;$finish;

end

endmodule

Experiment 9:

Write HDL implementation for a 4-bit right shift register using behavioral model. Simulate the same using behavioral model and depict the timing diagram for valid inputs.

Main Module:

module rshift(input clk,input clrb,input sdr,output reg [3:0]q);

always @(posedge(clk),negedge(clrb))

if(~clrb)

q<=4'b0000;

else

q<={sdr,q[3:1]};

endmodule

Test Module:

`include "rshift.v"

module test_rshift;

reg clk,clrb,sdr;

wire [3:0]q;

rshift rs(clk,clrb,sdr,q);

initial

begin

$dumpfile("shift.vcd");

$dumpvars(0,test_rshift);

clk=1;

clrb=0;

sdr=1;

#100

clrb=1;

sdr=1;

#150

sdr=0;

#200

$finish;

end

always #5 clk=~clk;

endmodule

Experiment 10:

Write HDL implementation for a 3-bit up-counter using behavioral model. Simulate the same using behavioral model and depict the timing diagram for valid inputs

Main Module:

module counter2 ( count,rst,clk);

input rst, clk;

output reg [2:0] count;

always @(posedge (clk))

if (rst)

count<= 3'b000;

else

count<= count + 1;

endmodule

Test Module:

`include "counter2.v"

module testmod;

reg r,c;

wire  [2:0] ct;

counter2 countbeh (ct,r,c);

initial

begin

$dumpfile("count.vcd");

$dumpvars(0,testmod);

r =1;

c=0;

#100 r=0;

#200 $finish;

end

always #5 c=~c;

endmodule

DATA FLOW MODELING

Experiment 11:

Write HDL implementation for AND/OR/NOT gates using data flow model. Simulate the same using data flow model and depict the timing diagram for valid inputs.

Main Module:

module gates(input a,b,output [2:0]y);

assign y[2]=a&b;

assign y[1]=a|b;

assign y[0]=~a;

endmodule

Test Module:

`include "gates.v"

module test_gates;

wire [2:0]y;

reg a,b;

gates dut(.y(y),.a(a),.b(b));

initial

begin

$dumpfile("gates.vcd");

$dumpvars(0,test_gates);

a=1'b0;

b=1'b0;

#50;

a=1'b0;

b=1'b1;

#50;

a=1'b1;

b=1'b0;

#50;

a=1'b1;

b=1'b1;

#50;

end

endmodule

Experiment 12:

Write HDL implementation for a 3-bit full adder using data flow model. Simulate the same using data flow model and depict the timing diagram for valid inputs.

Main Module:

module fa1(a,b,cin,s,cout);

input a,b,cin;

output s,cout;

assign s=a^b^cin;

assign cout=(a&b)|(b&cin)|(cin&a);

endmodule

Test Module:

`include "fulladd.v"

module test_fa;

reg a,b,cin;

wire s,cout;

fulladd f1(a,b,cin,s,cout);

initial

begin

$dumpfile("fa.vcd");

$dumpvars(0,test_fa);

a=1;b=1;cin=0;

#5

a=1;b=1;cin=1;

#5

a=0;b=1;cin=0;

#100 $finish;

end

endmodule

Signature of the staff in-charge